Welcome to Rockin Boxes.  This software will run with virtually any graphics,

but runs best using the VGA 640x480x16 mode. When loading it will automatically

check for a math co-processor, but it is not necessary to run the program.

If you must run this software in a mode other than what runme selects for you

(I have seen a few people with CGA monitors hooked up to EGA cards), just type

rockinb <adaptor_type> or rockmath <adaptor_type>, for example:  rockinb ega,

rockmath herc, or rockinb cga.

This is not Freeware, it is Shareware.  Try it out, if you use it please

register.  Registered users will get an uncrippled copy of the latest

version, lots of new drivers, and free support.

Coming soon...

I am currently sifting through a method to simulate bandpass enclosures,

hopefully soon this will be added.  The next big project will be a

crossover designer which will give you the same kind of response curves

that Rockin' Boxes does.

A note on the report printing (hit P while viewing the Watts vs. Spl graph);

it is written to work on any Epson compatable graphics printer.  This should

work on 95% of the dot matrix printers out there, but you may have to set

your printer to that emulation mode.  The registered version will have

additional printer support.

Rockin Boxes will automatically give you the best possible design for your

driver going by the numbers.  Numbers are not what you listen to though.

Design your system the way you want.

I've found that because of the bass boost created by the acoustics of most

vehicles that closed boxes tend to give the best low bass.  A typical car will

have a 12db/octave boost below about 80hz.  Ported systems will give lots of

nice midbass (40-70hz) which for most listening is fine, even the lowest pipe

organ only hits about 35-40hz.  One nice way to get a ported system to sound

like a closed box is to play with the box size and lower the port frequency.

The slope will be like a closed box with a few extra decibels of efficiency.

The trade off is sometimes the duct length gets to be enormous.  For home

systems I've found that ported systems work best,  you don't usually get the

free bass, unless you have a small room, and a typical ported system will give

pretty good response down to 40hz.  That is good enough to play 99.99% of the

music out there.  But then again a 20hz rumble sounds really cool as the light

fixtures fall out of your ceiling.

Be generous with the size of your duct.  Usually the bigger the better.

Smaller ports tend to wistle and degrade performance at higher power levels.

I recommend at least 3" diameter for 8" woofers, 4" for 10's, 5" for 12's, and

6 - 8" for 15's.  A port is just a duct that is the length of the material

used to build the box.  I don't recommend true port systems, it gets back to

the bigger the better.  The hole is usually pretty small compared to the

surface area of the driver.

Infinite baffle systems can also be simulated.  Choose a closed box and change

the volume to about 5 to 10 times the Vas of the driver this will give the

response of the driver using an infinite baffle.  This is also good for really

low bass response in a car.  The slopes sometimes get as low as 3db/octave so

even a driver with a high cutoff will still have excellent lower bass.  You do

lose some power handling ability though and run in to some excursion problems

at lower frequencies.  Drivers with a Qts of about .6 or greater are ideal for

infinite baffles.

Remember...

Practice Safe Sound!

If you don't like it, change it!

Have fun and enjoy the music because that's what it's all about!